W25Q32RV

# 8.   INSTRUCTIONS

The Standard/Dual/Quad SPI Instruction set of the W25Q32RV consists of 40 basic Instructions that are

fully controlled through the SPI bus (see Instruction Set Table1-2). Instructions are initiated with the falling edge of Chip Select (/CS). The first byte of data clocked into the DI input provides the Instruction code. Data on the DI input is sampled on the rising edge of clock with most significant bit (MSB) first.

The QPI Instruction set of the W25Q32RV consists of 28 basic Instructions that are fully controlled through the SPI bus (see Instruction Set Table 3). Instructions are initiated with the falling edge of Chip Select (/CS). The first byte of data clocked through IO[3:0] pins provides the Instruction code. Data on all four IO pins are sampled on the rising edge of clock with most significant bit (MSB) first. All QPI Instructions, addresses, data and dummy bytes are using all four IO pins to transfer every byte of data with every two serial clocks (CLK).

Instructions vary in length from a single byte to several bytes and may be followed by address bytes, data bytes, dummy bytes (don’t care), and in some cases, a combination. Instructions are completed with the rising edge of edge /CS. Clock relative timing diagrams for each Instruction are included in Figures 5 through 57. All read Instructions can be completed after any clocked bit. However, all Instructions that Write, Program or Erase must complete on a byte boundary (/CS driven high after a full 8-bits have been clocked) otherwise the Instruction will be ignored. This feature further protects the device from inadvertent writes. Additionally, while the memory is being programmed or erased, or when the Status Register is being written, all Instructions except for Read Status Register will be ignored until the program or erase cycle has completed.

## 8.1   Device ID and Instruction Set Tables

Manufacturer and Device Identification

| MANUFACTURER ID | (MF7 - MF0) |  |
| --- | --- | --- |
| Winbond Serial Flash | EFh |  |
| Device ID | (ID7 - ID0) | (ID15 - ID0) |
| Instruction | ABh, 90h, 92h, 94h | 9Fh |
| W25Q32RV-Q | 15h | 4016h |

- 19 -

W25Q32RV

Instruction Set Table 1 (Standard SPI Instructions) (1)

| Data Input Output |  | Byte 1 |  | Byte 2 |  | Byte 3 |  | Byte 4 |  | Byte 5 |  | Byte 6 |  | Byte 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Clock(1-1-1) |  | 8 |  | 8 |  | 8 |  | 8 |  | 8 |  | 8 |  | 8 |
| Write Enable |  | 06h |  |  |  |  |  |  |  |  |  |  |  |  |
| Volatile SR Write Enable |  | 50h |  |  |  |  |  |  |  |  |  |  |  |  |
| Write Disable |  | 04h |  |  |  |  |  |  |  |  |  |  |  |  |
| Release Power-down / ID |  | ABh |  | Dummy |  | Dummy |  | Dummy |  | (ID7-ID0)(2) |  |  |  |  |
| Manufacturer/Device ID |  | 90h |  | Dummy |  | Dummy |  | 00h |  | (MF7-MF0) |  | (ID7-ID0) |  |  |
| JEDEC ID |  | 9Fh |  | (MF7-MF0) |  | (ID15-ID8) |  | (ID7-ID0) |  |  |  |  |  |  |
| Read Unique ID |  | 4Bh |  | Dummy |  | Dummy |  | Dummy |  | Dummy |  | (UID63-0) |  |  |
| Read Data |  | 03h |  | A23-A16 |  | A15-A8 |  | A7-A0 |  | (D7-D0) |  |  |  |  |
| Fast Read |  | 0Bh |  | A23-A16 |  | A15-A8 |  | A7-A0 |  | Dummy |  | (D7-D0) |  |  |
| Page Program |  | 02h |  | A23-A16 |  | A15-A8 |  | A7-A0 |  | D7-D0 |  | D7-D0(3) |  |  |
| Sector Erase (4KB) |  | 20h |  | A23-A16 |  | A15-A8 |  | A7-A0 |  |  |  |  |  |  |
| Block Erase (32KB) |  | 52h |  | A23-A16 |  | A15-A8 |  | A7-A0 |  |  |  |  |  |  |
| Block Erase (64KB) |  | D8h |  | A23-A16 |  | A15-A8 |  | A7-A0 |  |  |  |  |  |  |
| Chip Erase |  | C7h/60h |  |  |  |  |  |  |  |  |  |  |  |  |
| Read Status Register-1 |  | 05h |  | (S7-S0)(2) |  |  |  |  |  |  |  |  |  |  |
| Write Status Register-1 |  | 01h |  | (S7-S0) |  |  |  |  |  |  |  |  |  |  |
| Read Status Register-2 |  | 35h |  | (S15-S8)(2) |  |  |  |  |  |  |  |  |  |  |
| Write Status Register-2 |  | 31h |  | (S15-S8) |  |  |  |  |  |  |  |  |  |  |
| Read Status Register-3 |  | 15h |  | (S23-S16)(2) |  |  |  |  |  |  |  |  |  |  |
| Write Status Register-3 |  | 11h |  | (S23-S16) |  |  |  |  |  |  |  |  |  |  |
| Read SFDP Register |  | 5Ah |  | A23-A16 |  | A15-A8 |  | A7-A0 |  | dummy |  | (D7-0) |  |  |
| Erase Security Register(4) |  | 44h |  | A23-A16 |  | A15-A8 |  | A7-A0 |  |  |  |  |  |  |
| Program Security Register(4) |  | 42h |  | A23-A16 |  | A15-A8 |  | A7-A0 |  | D7-D0 |  | D7-D0(3) |  |  |
| Read Security Register((4) |  | 48h |  | A23-A16 |  | A15-A8 |  | A7-A0 |  | Dummy |  | (D7-D0) |  |  |
| Erase / Program Suspend |  | 75h |  |  |  |  |  |  |  |  |  |  |  |  |
| Erase / Program Resume |  | 7Ah |  |  |  |  |  |  |  |  |  |  |  |  |
| Power-down |  | B9h |  |  |  |  |  |  |  |  |  |  |  |  |
| Set Read Parameters |  | C0h |  | P7-P0 |  |  |  |  |  |  |  |  |  |  |
| Enter QPI Mode |  | 38h |  |  |  |  |  |  |  |  |  |  |  |  |
| Enable Reset |  | 66h |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset Device |  | 99h |  |  |  |  |  |  |  |  |  |  |  |  |

Publication Release  Date:  November 13, 2025 
- 20 -	-Revision E

W25Q32RV

Instruction Set Table 2 (Dual/Quad SPI Instructions) (1)

| Data Input Output | Byte 1 | Byte 2 | Byte 3 | Byte 4 | Byte 5 | Byte 6 | Byte 7 | Byte 8 | Byte 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Clock(1-1-2) | 8 | 8 | 8 | 8 | 4 | 4 | 4 | 4 | 4 |
| Fast Read Dual Output | 3Bh | A23-A16 | A15-A8 | A7-A0 | Dummy | Dummy | (D7-D0)(6) | … |  |
| Number of Clock(1-2-2) | 8 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| Fast Read Dual I/O | BBh | A23-A16(5) | A15-A8(5) | A7-A0(5) | M7-M0 | (D7-D0)(6) | … |  |  |
| Mftr./Device ID Dual I/O | 92h | A23-A16(5) | A15-A8(5) | 00(5) | Dummy(17) | (MF7-MF0) (6) | (ID7-ID0)(6) |  |  |
| Number of Clock(1-1-4) | 8 | 8 | 8 | 8 | 2 | 2 | 2 | 2 | 2 |
| Quad Input Page Program | 32h | A23-A16 | A15-A8 | A7-A0 | (D7-D0)(8) | (D7-D0)(3) | … |  |  |
| Fast Read Quad Output | 6Bh | A23-A16 | A15-A8 | A7-A0 | Dummy | Dummy | Dummy | Dummy | (D7-D0)(9) |
| Number of Clock(1-4-4) | 8 | 2(7) | 2(7) | 2(7) | 2 | 2 | 2 | 2 | 2 |
| Mftr./Device ID Quad I/O | 94h | A23-A16 | A15-A8 | 00 | Dummy(13) | Dummy | Dummy | (MF7-MF0) | (ID7-ID0) |
| Fast Read Quad I/O | EBh | A23-A16 | A15-A8 | A7-A0 | M7-M0(13) | Dummy(11) | Dummy(11) | (D7-D0) | … |
| Set Burst with Wrap | 77h | Dummy | Dummy | Dummy | W7-W0 |  |  |  |  |

- 21 -

W25Q32RV

Instruction Set Table 3 (QPI Instructions)(10)

| Data Input Output | Byte 1 |  | Byte 2 |  | Byte 3 |  | Byte 4 |  | Byte 5 |  | Byte 6 |  | Byte 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Clock (4-4-4) | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 2 |
| Write Enable | 06h |  |  |  |  |  |  |  |  |  |  |  |  |
| Volatile SR Write Enable | 50h |  |  |  |  |  |  |  |  |  |  |  |  |
| Write Disable | 04h |  |  |  |  |  |  |  |  |  |  |  |  |
| Release Power-down / ID | ABh |  | Dummy |  | Dummy |  | Dummy |  | (ID7-ID0)(2) |  |  |  |  |
| Manufacturer/Device ID | 90h |  | Dummy |  | Dummy |  | 00h |  | (MF7-MF0) |  | (ID7-ID0) |  |  |
| JEDEC ID | 9Fh |  | (MF7-MF0) |  | (ID15-ID8) |  | (ID7-ID0) |  |  |  |  |  |  |
| Set Read Parameters | C0h |  | P7-P0 |  |  |  |  |  |  |  |  |  |  |
| Page Program | 02h |  | A23-A16 |  | A15-A8 |  | A7-A0 |  | D7-D0(9) |  | D7-D0(3) |  | … |
| Sector Erase (4KB) | 20h |  | A23-A16 |  | A15-A8 |  | A7-A0 |  |  |  |  |  |  |
| Block Erase (32KB) | 52h |  | A23-A16 |  | A15-A8 |  | A7-A0 |  |  |  |  |  |  |
| Block Erase (64KB) | D8h |  | A23-A16 |  | A15-A8 |  | A7-A0 |  |  |  |  |  |  |
| Chip Erase | C7h/60h |  |  |  |  |  |  |  |  |  |  |  |  |
| Read Status Register-1 | 05h |  | (S7-S0)(2) |  |  |  |  |  |  |  |  |  |  |
| Write Status Register-1 | 01h |  | (S7-S0) |  |  |  |  |  |  |  |  |  |  |
| Read Status Register-2 | 35h |  | (S15-S8)(2) |  |  |  |  |  |  |  |  |  |  |
| Write Status Register-2 | 31h |  | (S15-S8) |  |  |  |  |  |  |  |  |  |  |
| Read Status Register-3 | 15h |  | (S23-S16)(2) |  |  |  |  |  |  |  |  |  |  |
| Write Status Register-3 | 11h |  | (S23-S16) |  |  |  |  |  |  |  |  |  |  |
| Power-down | B9h |  |  |  |  |  |  |  |  |  |  |  |  |
| Enable Reset | 66h |  |  |  |  |  |  |  |  |  |  |  |  |
| Reset Device | 99h |  |  |  |  |  |  |  |  |  |  |  |  |
| Exit QPI Mode | FFh |  |  |  |  |  |  |  |  |  |  |  |  |
| Data Input Output | Byte 1 |  | Byte 2 |  | Byte 3 |  | Byte 4 |  | Byte 5 |  | Byte 6 |  | Byte 7 |
| Number of Clock (4-4-4) | 2 |  | 2 |  | 2 |  | 2 |  | 2 |  | 4 |  | 2 |
| Fast Read | 0Bh |  | A23-A16 |  | A15-A8 |  | A7-A0 |  | Dummy |  | Dummy(11) |  | (D7-D0) |
| Burst Read with Wrap | 0Ch |  | A23-A16 |  | A15-A8 |  | A7-A0 |  | Dummy |  | Dummy(11) |  | (D7-D0) |
| Fast Read Quad I/O | EBh |  | A23-A16 |  | A15-A8 |  | A7-A0 |  | M7-M0(13) |  | Dummy(11) |  | (D7-D0) |

Publication Release  Date:  November 13, 2025 
- 22 -	-Revision E

W25Q32RV

Notes:

1. Data bytes are shifted with Most Significant Bit first. Byte fields with data in parenthesis “( )” indicate data output

from the device on either 1, 2 or 4 IO pins.

2. The Status Register contents and Device ID will repeat continuously until /CS terminates the Instruction.

3. At least one byte of data input is required for Page Program, Quad Page Program and Program Security Registers,

up to 256 bytes of data input. If more than 256 bytes of data are sent to the device, the addressing will wrap to  the beginning of the page and overwrite previously sent data.

4. Security Register Address:

Security Register 1:

Security Register 2:

Security Register 3:

A23-16 = 00h;

A23-16 = 00h;

A23-16 = 00h;

A15-8 = 10h;

A15-8 = 20h;

A15-8 = 30h;

A7-0 = byte address

A7-0 = byte address

A7-0 = byte address

5. Dual SPI address input format:

IO0 = A22, A20, A18, A16, A14, A12, A10, A8   A6, A4, A2, A0, M6, M4, M2, M0

IO1 = A23, A21, A19, A17, A15, A13, A11, A9   A7, A5, A3, A1, M7, M5, M3, M1 6. Dual SPI data output format:

IO0 = (D6, D4, D2, D0)

IO1 = (D7, D5, D3, D1)

7. Quad SPI address input format:

IO0 = A20, A16, A12, A8,

IO1 = A21, A17, A13, A9,

A4, A0, M4, M0

A5, A1, M5, M1

Set Burst with Wrap input format:

IO0 = x, x, x, x, x, x, W4, x

IO1 = x, x, x, x, x, x, W5, x

IO2 = A22, A18, A14, A10, A6, A2, M6, M2	IO2 = x, x, x, x, x, x, W6, x

IO3 = A23, A19, A15, A11, A7, A3, M7, M3

8. Quad SPI data input/output format:

IOO = (D4, DO, . . ..)

IO1 = (D5, D1, . . ..)

IO2 = (D6, D2, . . ..)

IO3 = (D7, D3, . . ..)

9. Fast Read Quad I/O data output format:

IO0 = (x, x, x, x, D4, D0, D4, D0)

IO1 = (x, x, x, x, D5, D1, D5, D1)  	IO2 = (x, x, x, x, D6, D2, D6, D2)  	IO3 = (x, x, x, x, D7, D3, D7, D3)

IO3 = x, x, x, x, x, x, x,

x

10.    QPI Instruction, Address, Data input/output format:

CLK # 0	1	2	3	4	5	6	7	8	9	10   11

IO0 = C4, C0,

IO1 = C5, C1,

IO2 = C6, C2,

IO3 = C7, C3,

A20, A16,

A21, A17,

A22, A18,

A23, A19,

A12, A8,

A13, A9,

A14, A10,

A15, A11,

A4, A0,

A5, A1,

A6, A2,

A7, A3,

D4, D0,

D5, D1,

D6, D2,

D7, D3,

D4, D0

D5, D1

D6, D2

D7, D3

11.

12.

13.

The number of dummy clocks for QPI Fast Read, QPI/SPI Fast Read Quad I/O & QPI Burst Read with Wrap is

controlled by read parameter P7 - P4.

The wrap around length for QPI Burst Read with Wrap is controlled by read parameter P3 - P0.

The first dummy is M7-M0 should be set to Fxh, if Read Instruction bypass mode doesn’t use.

- 23 -